General System Information XE "General System Information" \b 
1.1.1 The following chapter details general information. Listed below are driver files you will find on the driver CD and information on the Microsoft Registry settings.

1.2 File lists XE "File lists" \b 
1.2.1 The following page summerises the files used by the PodMng.exe software package.

nomenclature:

[windows] indicates current windows directory (e.g. C:\WINDOWS)

[system] indicates you windows system directory (e.g. C:\WINDOWS\SYSTEM)

[windows]\podmng.exe

NetPod manager interface

 [system]\netpod.dll

NetPod driver

1.2.2 In addition there may be drivers specific to your SCADA, MMI or data processing software.
1.3 Registry settings XE "Registry settings" \b 
1.3.1 Registry setting are placed under the HKEY_CURRENT_USER\Software\NetPod key. You can use Regedit to modify the settings (from the start menu, select run, and type Regedit) The keys are listed as follows:

Access

User XE "User" \b  access mode, e.g. user or operator






if this key is set to UA5098, then user access mode is selected. 

1.3.2 The user has full access to configure the NetPod.. All other keys default to operator mode. Operator XE "Operator" \b s can not configure NetPods

1.3.3 The subkey InitCommands XE "subkey InitCommands" \b  contains commands to be run when the NetPod driver software is first run. there are a sequence of commands as follows:

InitCommands\Command1

InitCommands\Command2

InitCommands\Command3
etc.

These command are run sequentially, and the keys are string values that can be any of the following:

SCANNET XE "SCANNET" \b 

Scan the network on initialisation

SCANNET XE "SCANNET" \b  D

Scan the network on initialisation, and display the progress bar

SCANPORT 1 XE "SCANPORT 1" \b 

scan comm port 1 (COM1)

SCANPORT 2

scan comm port 2 (COM2) etc.

STARTRUN XE "STARTRUN:Prevent error log screen being viewed by user." 

start acquisition operations

NOPACKERR

Prevent the error log screen from being viewed in driver.

























































